Electronics and Electrical Communications Engineering
Department

Faculty of Engineering

Cairo University

Submitted to: Dr. Mahmoud El-Hadidi

AquaVision: Scalable IoT-Based Smart Aquaculture Management System with Individual Fish Tracking

Initial Report submitted for course ELC4015 “Selected Topics in Communications: Internet of Things“

4th Year

1st Semester - Academic Year 2025/2026

Prepared by:

Team 3

| NAME | SECTION | ID |
| --- | --- | --- |
| Yousef Khaled Omar Mahmoud | 4 | 9220984 |
| Mohammed Hatem Ahmed Mahmoud | 3 | 9220676 |
| Ahmed Mohamed Ahmed Soltan | 1 | 9220080 |

Submission Date: 29 November 2025

# Table of Contents

1-	Table of Contents	2

2-	Table of Figures	2

3-	List of tables	3

4-	Executive Summary	4

5-	Name of IoT Vertical Application	4

6-	Functions Performed by the IOT System	4

6.1	Individual Fish Tracking via LF-RFID	4

6.2	Multi-Tank Water Quality Monitoring	5

6.3	Hierarchical Data Aggregation (Star Topology)	6

6.4	AI-Powered Water Quality Optimization	7

6.5	Predictive Disease Detection and Prevention	7

6.6	Automated Feeding Optimization	8

6.7	Dual User Interface System	9

6.8	Scalable Commercial Deployment	10

7-	TYPE OF DATA COLLECTED	11

7.1	Per-Tank Sensor Data	11

2.2 Fish Tracking Data (LF-RFID)	12

2.3 Collector Aggregated Data	13

8-	Detailed Schematic Diagram for the IoT Application	15

8.1	System Overview	15

8.2	Layer 1: Perception Layer (Physical/Sensing Layer)	18

8.3	Layer 3: Application Layer (Processing/Service Layer)	18

8.4	Lessons Learned and Recommendations	19

9-	References:	19

9.1	 Appendix A: Deployment Overview	19

9.2	Appendix B: Acronyms and Abbreviations	19

2.4 Central Node to Cloud Data	21

# Table of Figures

Figure 3 Comprehensive System Architecture of LoRaWAN Smart Campus IoT Deployment showing perception, network (integrated NS), application (on-premise) and user layers (drawn by student based on [1])[3]	17

# List of tables

Table 1 Per-Tank Environmental Sensor Data Specifications	11

Table 2 Sensor-Level Functions[1]	18

Table 3 Overview of the LoRaWAN Smart Campus Deployment[1]	19

Table 4 Acronyms and Abbreviations[1], [2]	19

# Executive Summary

AquaVision is a comprehensive Internet of Things (IoT) solution designed for commercial-scale smart aquaculture, built on a modular, four-tier hierarchical architecture that allows for full customization based on farm size and complexity. While this report details a representative installation featuring 50 sensor nodes distributed across the farm, the design is inherently scalable to meet varying operational demands. The system's goal is to optimize farm productivity, reduce operational costs, and improve fish health outcomes. At the base, sensor nodes continuously monitor critical environmental parameters (temperature, pH, dissolved oxygen) and employ Low-Frequency Radio-Frequency Identification (LF-RFID) to track 7,500 fish individually for unprecedented behavioral and growth insights.

The Network Layer aggregates this massive dataset via LoRaWAN from 5 collector nodes, capitalizing on the protocol’s long-range and ultra-low power consumption, and ensures reliable, low-latency transmission to the central controller through wired Ethernet connections. The Edge Computing Layer, built around a Raspberry Pi 5 central node, provides the system's local intelligence and resilience. This node features a 10.1-inch touchscreen dashboard for on-site managers and performs edge AI inference for immediate control actions. Bidirectional communication with the remote AWS IoT Cloud is established using the secure MQTT-TLS protocol over 4G LTE. Leveraging the Cloud Application Layer, advanced AI algorithms analyze historical and real-time data using services like AWS SageMaker and RDS to optimize water quality, predict feeding schedules, and facilitate early disease outbreak detection, thereby driving critical, data-driven decisions.

End-users interact with the system through two intuitive interfaces: the local touchscreen dashboard for real-time manual control, and a mobile application for remote oversight. The mobile app’s centerpiece is an AI-powered chatbot that supports natural language queries for instant status updates and control commands, complemented by immediate push notifications and SMS alerts for critical events. AquaVision represents a transformative approach to sustainable aquaculture, positioning fish farms for increased profitability and environmental sustainability by significantly reducing manual labor and minimizing resource waste through continuous, AI-driven analytics.

# Name of IoT Vertical Application

AquaVision operates squarely within the Smart Agriculture vertical, specifically focusing on Smart Aquaculture and Precision Fish Farming. The system's primary goal is to transform traditional commercial fish farms into highly efficient, data-driven operations by continuously monitoring critical biological and environmental parameters, such as water quality and individual fish welfare. The deployment aims to optimize resource consumption, maximize yield, and ensure the environmental sustainability of modern fish farming practices, positioning it as a key application in the future of food technology [1].

# Functions Performed by the IOT System

## Individual Fish Tracking via LF-RFID

AquaVision implements a revolutionary fish tracking system using passive Low-Frequency Radio-Frequency Identification (LF-RFID) technology operating at 125 kHz. Each fish in the farm receives a biocompatible glass tube implant (12mm × 2mm dimensions) containing a passive RFID transponder during initial stocking procedures performed by trained veterinarians.

RDM6300 RFID readers integrated into each tank's sensor node continuously scan for tagged fish within a 5-10 cm detection radius. When a fish swims near the reader antenna (typically positioned at feeding zones or circulation areas), the system captures the fish's unique identification number, timestamp, and signal strength (RSSI), creating a comprehensive movement database.

Key Capabilities:

- Growth Monitoring: By correlating RFID detections with manual weight measurements during periodic handling, the system tracks individual fish growth rates and identifies slow-growing or potentially diseased individuals requiring intervention.

- Feeding Behavior Analysis: Detection frequency at feeding zones indicates appetite levels and feeding competition dynamics, enabling optimization of feeding schedules and quantities to reduce waste while maximizing growth.

- Mortality Detection: Fish that show no RFID detections for extended periods (typically 24-48 hours depending on tank size and fish species) trigger mortality alerts, allowing prompt removal of dead fish to prevent water quality degradation.

- Movement Pattern Analysis: Unusual movement patterns, such as increased surface activity or lethargic behavior (reduced detection frequency), can indicate environmental stress, disease onset, or water quality issues requiring immediate attention.

- Inventory Management: Automated fish counting provides real-time inventory data, eliminating manual counting errors and enabling precise harvest planning and restocking decisions.

This individual tracking capability represents a significant advancement over traditional aquaculture monitoring systems that only measure aggregate environmental parameters without insight into individual fish welfare and behavior.

## Multi-Tank Water Quality Monitoring

AquaVision continuously monitors four critical water quality parameters across all 50 tanks simultaneously, ensuring optimal environmental conditions for fish health and growth:

- Temperature Monitoring: DS18B20 waterproof digital temperature sensors measure water temperature with ±0.5°C accuracy every 5 minutes. The system maintains species-specific optimal temperature ranges (typically 25-30°C for tilapia, 24-28°C for catfish) and triggers cooling/heating actions when thresholds are exceeded. Temperature data correlates with dissolved oxygen levels and fish metabolic rates, informing feeding and aeration decisions.

- pH Level Monitoring: Analog pH sensors track water acidity/alkalinity every 10 minutes with ±0.1 pH unit accuracy. Fish health critically depends on maintaining species-appropriate pH levels (typically 6.5-8.5 for most species). Deviations trigger alerts and guide water treatment interventions such as lime addition (raise pH) or carbon dioxide injection (lower pH).

- Dissolved Oxygen (DO) Monitoring: Galvanic or optical dissolved oxygen sensors measure oxygen concentration every 5 minutes with ±0.5 mg/L accuracy. DO levels below 5 mg/L cause stress; below 3 mg/L causes fish mortality. The system automatically activates aerators when DO drops below 7 mg/L, maintaining a safety margin and ensuring continuous oxygen availability for fish respiration.

- Water Level Monitoring: HC-SR04 ultrasonic sensors mounted above each tank measure water depth every 10 minutes with ±1 cm accuracy. Low water levels indicate evaporation or leaks requiring attention; high levels suggest pump malfunctions or overflow risks. Automated pumps maintain target water levels, ensuring proper tank hydraulics and filtration system operation.

The simultaneous monitoring of all 50 tanks enables comparative analysis, identifying systemic issues (affecting multiple tanks) versus localized problems (single tank anomalies), accelerating troubleshooting and reducing response times.

## Hierarchical Data Aggregation (Star Topology)

AquaVision employs a carefully designed hierarchical architecture that balances system scalability, power efficiency, and communication reliability through a star network topology at each hierarchical level.

- Sensor Node Layer: Each of the 50 sensor nodes operates independently, collecting data from attached sensors and RFID readers. Nodes use ultra-low-power sleep modes between measurements, waking only to read sensors (30 seconds every 5 minutes) and transmit data packets (2-3 seconds per transmission), achieving multi-year battery life from a single 18650 Li-ion cell.

- Collector Node Layer: Five collector nodes (Raspberry Pi Zero W with LoRa HAT modules) serve as LoRaWAN gateways, each managing 10 sensor nodes in a star configuration. Collector nodes remain continuously powered via wall adapters, performing several critical functions:

- Data Aggregation: Receiving and buffering data packets from 10 sensor nodes, reducing the number of connections the central node must manage from 50 to 5.

- Local Processing: Basic data validation, outlier detection, and timestamp synchronization to ensure data quality before forwarding to the central node.

- Fault Isolation: If one collector fails, only 10 tanks are affected; the remaining 40 tanks continue normal operation, ensuring system resilience.

- Central Node Layer: The Raspberry Pi 5-based central node receives aggregated data from all 5 collectors via wired Ethernet connections (one RJ45 cable per collector). Ethernet provides gigabit bandwidth, sub-millisecond latency, and immunity to wireless interference, ensuring reliable real-time control for critical applications like emergency aerator activation

Star Topology Justification:

The deliberate choice of star topology over mesh networks (common in systems like ZigBee) provides several advantages:

- Simplified Routing: Direct point-to-point communication eliminates complex mesh routing algorithms, reducing software complexity and potential failure modes.

- Predictable Latency: Star topology guarantees single-hop communication with consistent, minimal latency, critical for real-time control applications.

- Power Efficiency: Sensor nodes transmit only to their collector (not relaying for other nodes), minimizing transmission time and power consumption.

- Scalability: Adding new sensor nodes requires only registering them with the nearest collector; no mesh re-optimization needed.

- Fault Tolerance: Collector failures are isolated; nodes do not depend on neighboring nodes for connectivity.

## AI-Powered Water Quality Optimization

AquaVision's cloud-based AI engine continuously analyzes sensor data alongside external factors (weather forecasts, seasonal patterns, fish growth stage) to dynamically optimize water quality parameters and predict future conditions.

Machine Learning Models:

- Water Quality Prediction Model: A time-series forecasting model (LSTM neural network) trained on historical data predicts temperature, pH, and DO levels 6-24 hours ahead based on current readings, time of day, weather forecasts, and seasonal trends. Predictions enable proactive interventions (e.g., pre-cooling water before predicted hot afternoon) rather than reactive responses.

- Optimal Setpoint Recommendation: Reinforcement learning algorithms optimize water quality target setpoints dynamically rather than using fixed thresholds. For example, during rapid fish growth phases, the system may recommend slightly elevated DO levels (9-10 mg/L instead of standard 8 mg/L) to support increased metabolic demands, maximizing growth rates while managing energy costs.

- Anomaly Detection: Unsupervised learning algorithms (autoencoders) detect unusual patterns in sensor data that may indicate equipment malfunctions (stuck sensor readings), water quality events (sudden pH drops from feed decomposition), or disease outbreaks (temperature spikes from bacterial infections) before they become critical.

- Energy Optimization: AI models balance fish welfare requirements against energy costs by optimizing aerator runtime schedules. For example, running aerators at full capacity during cooler nighttime hours (when DO naturally drops) and reducing daytime aeration (when photosynthesis produces oxygen) minimizes electricity costs while maintaining safety margins.

- Adaptive Control: Unlike traditional fixed-threshold systems (e.g., "activate aerator if DO < 7 mg/L"), AI-driven control adapts to changing conditions. During feeding periods when fish consume more oxygen, the system preemptively increases aeration; during inactive nighttime periods with low oxygen demand, it conserves energy.

## Predictive Disease Detection and Prevention

Early disease detection is critical in aquaculture; delayed intervention can result in high mortality rates and significant economic losses. AquaVision combines environmental monitoring with fish behavior analysis to predict disease outbreaks before visible clinical signs appear.

Multi-Modal Analysis:

- Environmental Risk Factors: Certain water quality patterns correlate with increased disease susceptibility:

  - Prolonged elevated temperatures (>32°C) stress fish immune systems

  - pH fluctuations indicate ammonia/nitrite issues that harm fish gills

  - Low DO levels weaken fish, making them vulnerable to pathogens

The system assigns risk scores based on how long conditions remain suboptimal, triggering preventive alerts when cumulative risk exceeds thresholds.

- Behavioral Indicators: RFID tracking reveals behavioral changes that precede visible disease symptoms:

  - Reduced Activity: Sick fish show decreased movement (fewer RFID detections per day)

  - Surface Gathering: Fish gasping at the surface (increased detections near surface-mounted RFID readers) indicate respiratory distress

  - Feeding Reluctance: Reduced detections at feeding zones indicate appetite loss, an early disease symptom

- Predictive Models: Historical data from previous disease outbreaks trains classification models to recognize patterns preceding infections. When current data matches these patterns, the system issues early warnings with recommended actions (increase monitoring frequency, prepare medication, reduce feeding to minimize organic load).

- Preventive Interventions: Upon detecting elevated disease risk, the system recommends preventive measures:

  - Prophylactic salt treatments to reduce pathogen loads

  - Probiotic supplementation to strengthen fish immunity

  - Water exchange schedules to dilute pathogen concentrations

  - Reduced stocking density in affected tanks to minimize transmission

## Automated Feeding Optimization

Feed represents 50-70% of aquaculture operating costs; optimizing feeding efficiency directly impacts farm profitability. AquaVision's AI-driven feeding system minimizes waste while maximizing fish growth through data-driven feeding schedules.

Dynamic Feed Scheduling:

- Growth-Stage Adaptation: Feeding requirements change throughout the fish life cycle. Fingerlings (juvenile fish) require frequent small meals (4-6 times daily) with high-protein content, while market-size fish need less frequent feeding (2-3 times daily). The system adjusts feeding schedules automatically based on average fish age and growth rate data.

- Environmental Compensation: Water temperature directly affects fish metabolism and digestion. At optimal temperatures (27-29°C for tilapia), fish digest food efficiently and require normal rations. At lower temperatures (<24°C), metabolism slows; the system reduces feeding quantities to prevent uneaten feed accumulation and water quality degradation.

- Appetite-Based Adjustment: RFID detections at feeding zones reveal feeding intensity. High detection frequency during feeding indicates strong appetite; the system may increase next feeding quantity. Low detection frequency suggests satiation or stress; the system reduces feeding to prevent waste.

- Feed Conversion Ratio (FCR) Optimization: FCR measures feed efficiency (kg feed ÷ kg fish weight gain). Lower FCR indicates better efficiency. The system tracks FCR per tank, identifying underperforming tanks requiring investigation (disease, poor water quality, equipment malfunction) and high-performing tanks whose conditions can be replicated elsewhere.

- Automated Dispenser Control: Servo-motor-controlled feeders receive commands from the central node, dispensing precise feed quantities at scheduled times. Manual override capabilities allow farm managers to adjust feeding for special circumstances (medication mixing, pre-harvest fasting).

## Dual User Interface System

AquaVision recognizes that different users require different interfaces: on-site farm managers need immediate access to controls and detailed technical data, while remote farm owners need high-level overviews and the ability to check farm status conveniently.

Central Node Touchscreen Dashboard (On-Site Interface):

Raspberry Pi 5 drives a 10.1-inch capacitive touchscreen displaying a real-time dashboard built with React web framework, accessible via localhost web browser.

Dashboard Components:

- Farm Overview Panel: Total tanks (50), total fish count (sum of RFID-detected fish across all tanks), active alert count, system health indicators (all collectors online, cloud connectivity status, battery levels for wireless nodes).

- Tank Status Grid: 10×5 grid visualization with each cell representing one tank, color-coded by status:

  - Green: All parameters within optimal ranges

  - Yellow: One or more parameters approaching thresholds (warning)

  - Red: Critical condition requiring immediate action

  - Gray: No data received (communication failure or powered off)

- Detailed Tank View: Selecting a tank displays real-time gauges (analog-style circular gauges for temperature, pH, DO; linear gauge for water level), 24-hour trend graphs, current fish count, and recent RFID activity timeline.

- Manual Control Panel: Farm managers can manually override automated controls:

  - Emergency stop (shuts down all actuators)

  - Per-tank aerator on/off toggle

  - Feeding dispenser manual trigger

  - Water pump start/stop

- Alert Management: Prioritized list of active alerts with acknowledge/dismiss buttons, alert history log with timestamps and resolution notes, and alert statistics dashboard showing most frequent issues.

Mobile Application (Remote Owner Interface):

Flutter-based cross-platform mobile app (iOS/Android) connects to AWS cloud backend via HTTPS REST API and WebSocket for real-time updates.

App Features:

- AI Chatbot: Natural language interface powered by AWS Lex or OpenAI API, allowing conversational queries:

  - User: "How is my farm doing today?"

  - AI: "Your farm is performing well. All 50 tanks are online. 3 tanks have minor temperature warnings (29-30°C) but within safe limits. Average fish growth rate this week: 45g per fish. No critical alerts."

- Dashboard View: Simplified overview showing key metrics (total fish, average temperature/pH/DO, active alerts) and tank grid with same color-coding as central display.

- Push Notifications: Critical alerts (DO below 5 mg/L, equipment failures) trigger immediate push notifications even when app is closed. High-priority alerts also send SMS as backup notification channel.

- Remote Control: Limited remote control for non-critical functions (acknowledge alerts, adjust automated feeding schedules, view detailed tank data). Critical controls (emergency stop, manual aerator activation) require on-site touchscreen access for safety.

- Analytics: Historical reports with graphs showing growth trends, FCR over time, water quality stability metrics, and cost analysis (feed consumption, electricity usage).

## Scalable Commercial Deployment

AquaVision's architecture is designed from the ground up for commercial scalability, supporting farms ranging from 50 tanks (initial deployment) to hundreds or thousands of tanks across multiple farm sites.

Modular Expansion:

- Adding Sensor Nodes: New tanks require only:

- Install sensor node (ESP32-S3 + sensors + RFID reader + LoRa module) on tank

- Power on (node auto-configures and attempts to join nearest collector)

- Register node ID in central node configuration (automatic discovery possible) No changes to other nodes or system architecture required.

- Adding Collectors: When sensor node density exceeds 10 per collector, add new collector:

- Deploy Raspberry Pi Zero W with LoRa HAT in new zone

- Connect collector to central node via Ethernet

- Update central node routing table with new collector ID Sensor nodes automatically associate with strongest-signal collector.

- Adding Farms: Multiple farms can share a single AWS cloud infrastructure:

- Each farm has unique Farm ID in database

- Central nodes from all farms connect to same AWS IoT Core endpoint

- Mobile app users select which farm to view from account dashboard

- Cloud AI models can compare performance across farms, identifying best practices

- Geographic Distribution: Farms separated by large distances (different cities/countries) connect to geographically nearest AWS region to minimize latency. AWS global infrastructure includes regions in Middle East (Bahrain), Europe (Frankfurt, Ireland), and Asia (Mumbai, Singapore), ensuring <200ms latency worldwide.

- Cost Scaling: Per-tank hardware costs decrease with volume (economy of scale for bulk component purchases). Cloud costs scale linearly with data volume (AWS charges per GB stored, per million API requests) but remain low ($0.10-0.50 per tank per month for typical data volumes). Energy savings from AI optimization (10-20% reduction in aerator runtime) often exceed cloud service costs.

# TYPE OF DATA COLLECTED

## Per-Tank Sensor Data

Each of the 50 sensor nodes collects environmental data from sensors connected to the ESP32-S3 microcontroller. The following table specifies data format, units, size, and collection frequency for each parameter:

Table 1 Per-Tank Environmental Sensor Data Specifications

| Parameter | Data Format | Units | Size (Bytes) | Range | Accuracy | Collection Frequency | Critical Threshold | Purpose |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tank ID | Unsigned Integer (16-bit) | - | 2 | 1-65535 | Exact | Every reading | N/A | Tank identification for data routing |
| Timestamp | Unix Epoch (32-bit) | Seconds since 1970 | 4 | 0-4.29B | ±1 second | Every reading | N/A | Temporal correlation of measurements |
| Temperature | Float (32-bit IEEE 754) | °C (Celsius) | 4 | -55 to +125 | ±0.5°C | Every 5 minutes | >32°C (High)<br><24°C (Low) | Fish metabolic rate optimization |
| pH Level | Float (32-bit IEEE 754) | pH units | 4 | 0-14 | ±0.1 pH | Every 10 minutes | <6.5 (Low)<br>>8.5 (High) | Water chemistry balance for fish health |
| Dissolved Oxygen (DO) | Float (32-bit IEEE 754) | mg/L | 4 | 0-20 | ±0.5 mg/L | Every 5 minutes | <7 mg/L (Low)<br><5 mg/L (Critical) | Fish respiration requirements |
| Water Level | Float (32-bit IEEE 754) | cm | 4 | 0-200 | ±1 cm | Every 10 minutes | <30 cm (Low)<br>>180 cm (High) | Tank overflow/underflow prevention |
| Battery Voltage | Float (32-bit IEEE 754) | Volts | 4 | 0-4.2V | ±0.05V | Every reading | <3.3V (Low battery warning) | Node power status monitoring |
| Signal Strength (RSSI) | Signed Integer (8-bit) | dBm | 1 | -120 to 0 | ±2 dBm | Every reading | <-110 dBm (Poor connection) | Communication link quality |

Total Packet Size per Reading: 2 + 4 + 4 + 4 + 4 + 4 + 4 + 1 = 27 bytes

Collection Schedule:

- Temperature and DO: Every 5 minutes (critical parameters requiring frequent monitoring)

- pH and Water Level: Every 10 minutes (slower-changing parameters)

- All readings include Tank ID, Timestamp, Battery Voltage, and RSSI metadata

Daily Data Volume per Tank:

- 5-minute readings: (24 hours × 60 min ÷ 5 min) = 288 readings/day

- 10-minute readings: (24 hours × 60 min ÷ 10 min) = 144 readings/day

- Average: ~220 readings/day per tank

- Data volume: 220 readings × 27 bytes = 5.94 KB/tank/day

- Total for 50 tanks: 297 KB/day ≈ 0.3 MB/day ≈ 9 MB/month

Data Encoding: Sensor data is encoded in binary format for transmission efficiency over LoRaWAN (which has strict payload size limits of 51-222 bytes depending on spreading factor). The ESP32-S3 firmware constructs compact binary packets following this structure:

[Header: 2 bytes] [Tank ID: 2 bytes] [Timestamp: 4 bytes]

[Temp: 4 bytes] [pH: 4 bytes] [DO: 4 bytes] [Level: 4 bytes]

[Battery: 4 bytes] [RSSI: 1 byte] [Checksum: 2 bytes]

Total: 31 bytes (fits within smallest LoRaWAN payload size)

### 2.2 Fish Tracking Data (LF-RFID)

RFID readers continuously scan for tagged fish. Detection events are logged asynchronously (event-driven rather than periodic sampling) whenever a fish enters the reader's detection zone.

Table 2.2: Fish Tracking RFID Data Specifications

| Parameter | Data Format | Units | Size (Bytes) | Range/Description | Collection Trigger | Purpose |
| --- | --- | --- | --- | --- | --- | --- |
| Fish ID | String (12 chars ASCII) | - | 12 | Unique RFID tag UID (e.g., "3A4B5C6D7E8F") | When fish detected | Individual fish identification |
| Tank ID | Unsigned Integer (16-bit) | - | 2 | 1-50 | When fish detected | Current fish location |
| Detection Timestamp | Unix Epoch (32-bit) | Seconds | 4 | Current time | When fish detected | Movement timeline |
| Reader ID | Unsigned Integer (8-bit) | - | 1 | 1-255 | When fish detected | Which reader in tank (if multiple readers) |
| RSSI | Signed Integer (8-bit) | dBm | 1 | -60 to -20 dBm | When fish detected | Fish proximity estimation (closer = stronger signal) |
| Detection Duration | Unsigned Integer (16-bit) | Milliseconds | 2 | 10-5000 ms | When fish detected | How long fish remained in detection zone |

Total Packet Size per Detection: 12 + 2 + 4 + 1 + 1 + 2 = 22 bytes

Detection Frequency: RFID detection frequency depends on fish activity patterns:

- High Activity Periods (Feeding Time): 10-20 detections per fish per hour (fish congregate near feeders where RFID readers are often positioned)

- Normal Activity: 2-5 detections per fish per hour (fish swim throughout tank, occasionally passing near reader)

- Low Activity (Night): 0.5-2 detections per fish per hour (fish rest on tank bottom or corners)

Estimated Daily Data Volume (Fish Tracking):

- Average detections per fish: ~100 detections/day (varies by species behavior)

- Fish per tank: 150

- Detections per tank per day: 150 fish × 100 = 15,000 detections

- Data per tank: 15,000 × 22 bytes = 330 KB/tank/day

- Total for 50 tanks: 16.5 MB/day ≈ 495 MB/month

Storage Optimization: Raw RFID detection logs generate significant data volume. The system employs two-tier storage:

- Real-Time Edge Storage (Last 7 Days): Full-resolution detection logs stored in Central Node local SQLite database for immediate analysis

- Cloud Long-Term Storage (Historical): Aggregated summaries stored in AWS S3:

  - Per-fish daily summaries (total detections, average RSSI, time distribution)

  - Tank-level hourly summaries (total unique fish detected, movement heatmaps)

  - Reduces storage by ~95% while preserving analytical value

### 2.3 Collector Aggregated Data

Collector nodes receive data from 10 sensor nodes, perform local aggregation, and forward consolidated packets to the central node via Ethernet.

Table 2.3: Collector Node Aggregated Data Structure

| Parameter | Data Format | Units | Size (Bytes) | Frequency | Description |
| --- | --- | --- | --- | --- | --- |
| Collector ID | Unsigned Integer (8-bit) | - | 1 | Every transmission | Collector identification (1-5) |
| Timestamp | Unix Epoch (32-bit) | Seconds | 4 | Every transmission | Data collection timestamp |
| Number of Active Nodes | Unsigned Integer (8-bit) | Count | 1 | Every transmission | How many of 10 nodes successfully transmitted |
| Sensor Data Payload | Nested Array | Mixed | Variable (270-310 bytes) | Every 5 minutes | Contains all 10 sensor node readings |
| RFID Data Payload | Nested Array | Mixed | Variable (0-2000 bytes) | Batch every 10 minutes | Contains RFID detections since last transmission |
| Alert Flags | Bitfield (16-bit) | Flags | 2 | Every transmission | Per-tank critical alert status (1 bit per tank) |
| Collector Health | Struct | Mixed | 8 bytes | Every transmission | CPU load, memory usage, uplink status |

Transmission Frequency:

- Environmental Data: Every 5 minutes (synchronized with sensor node transmission schedule)

- RFID Data: Every 10 minutes (batched to reduce Ethernet traffic)

- Alerts: Immediate (out-of-band transmission if critical condition detected)

Daily Data Volume per Collector:

- Environmental data: 288 transmissions × 280 bytes avg = 80.6 KB/day

- RFID data: 144 batches × 500 bytes avg = 72 KB/day

- Total per collector: ~153 KB/day

- Total for 5 collectors: 765 KB/day ≈ 23 MB/month

Ethernet Communication: Collectors connect to central node via CAT6 Ethernet cables (up to 100m length). TCP/IP protocol ensures reliable delivery with automatic retransmission of lost packets. Each collector is assigned a static IP address (192.168.10.11 through 192.168.10.15) on a dedicated IoT subnet isolated from farm office network for security.

# References:

[1] Omar, Y. K.  AquaVision-Accessible-IoT-Aquaponics. GitHub. Available: https://github.com/youefkh05/AquaVision-Accessible-IoT-Aquaponics

Appendix B: Acronyms and Abbreviations

Table 4 summarizes all acronyms used throughout this report for quick reference.

Table 4 Acronyms and Abbreviations[1]

| Acronym | Full Form |
| --- | --- |
| ABP | Activation By Personalization |
| ADR | Adaptive Data Rate |
| AES | Advanced Encryption Standard |
| API | Application Programming Interface |
| AppEUI | Application Extended Unique Identifier |
| AppKey | Application Key |
| AppSKey | Application Session Key |
| CO₂ | Carbon Dioxide |
| CSS | Chirp Spread Spectrum |
| DevAddr | Device Address |
| DevEUI | Device Extended Unique Identifier |
| GDPR | General Data Protection Regulation |
| HTTPS | Hypertext Transfer Protocol Secure |
| HVAC | Heating, Ventilation, and Air Conditioning |
| IoT | Internet of Things |
| IP | Internet Protocol |
| ISP | Internet Service Provider |
| LoRa | Long Range |
| LoRaWAN | Long Range Wide Area Network |
| LPWAN | Low Power Wide Area Network |
| LTE | Long-Term Evolution |
| MAC | Media Access Control |
| MIC | Message Integrity Code |
| MQTT | Message Queuing Telemetry Transport |
| OTAA | Over-The-Air Activation |
| PER | Packet Error Rate |
| PIR | Passive Infrared |
| QoS | Quality of Service |
| RBAC | Role-Based Access Control |
| RF | Radio Frequency |
| SF | Spreading Factor |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| TX | Transmit / Transmission |
| RX | Receive / Reception |
| UDP | User Datagram Protocol |
| NwkSKey | Network Session Key |